The first concrete mix tested was class 47B, containing 1155 lb/yd^3 of cement, 290 lb/yd^3 of fly ash, a water ratio of 0.4, an air content of 6%, fine aggregate and coarse aggregate proportions of 1150 and 1750 lb/yd^3 respectively, and specific gravities of cement, fly ash, fine aggregates, and coarse aggregates at 3.15, 2.4, 2.65, and 2.7, respectively.

It has a resultant weight of fine aggregate at 549 lb, coarse aggregate at 728 lb, other aggregates at 110 lb and water at 578 lb.

The second concrete mix tested was class 47BR, containing 1155 lb/yd^3 of cement, 150 lb/yd^3 of fly ash, a water ratio of 0.42, an air content of 7%, fine aggregate, coarse aggregate, and other aggregate proportions of 1050, 1850, and 250 lb/yd^3 respectively, and specific gravities of cement, fly ash, fine aggregates, coarse aggregates, and other aggregates at 3.15, 2.4, 2.65, 2.7, and 2.68 respectively.

It has a resultant weight of fine aggregate at 594  lb, coarse aggregate at 780 lb, other aggregate at 205, and water at 548 lb.

The third concrete mix tested was class 40, containing 658 lb/yd^3 of cement, 132 lb/yd^3 of fly ash, a water ratio of 0.44, an air content of 6%, fine aggregate and coarse aggregate proportions of 1200 and 1700 lb/yd^3 respectively, and specific gravities of cement, fly ash, fine aggregates, and coarse aggregates at 3.15, 2.4, 2.65, and 2.7 respectively.

It has a resultant weight of fine aggregate at 1108 lb, coarse aggregate at 1496 lb, other aggregate at 0lb and water at 348 lb.

The final concrete mix tested was class D, containing 470 lb/yd^3 of cement, 180 lb/yd^3 of fly ash, a water ratio of 0.45, an air content of 4.5%, fine aggregate and coarse aggregate proportions of 1150 and 1950 lb/yd^3 respectively, and specific gravities of cement, fly ash, fine aggregates, and coarse aggregates at 3.15, 2.4, 2.65, and 2.7 respectively.

It has a resultant weight of fine aggregate at 1071 lb, coarse aggregate at 1858 lb, other aggregate at 0lband water at 292 lb.